Shortwave Radiation

SW = Sd (or Su)/ Coeff

Sd : Downward Shortwave (Thermopile, Unit:mV) (No 31 at Data File)

Su : Upward Shortwave (Thermopile, Unit : mV) (No 33)

Coeff : Calibration Coefficient (Unit : mV-1 W-1 m-2 )

Longwave Radiation

For Downward Longwave,

LW = Ld_Rnet / K. + 1000*(Ld_Case_TR) + 1000*C (Ld_Dome_TR - Ld_Case_TR)

Ld_Rnet : Thermopile output (mV) (No. 38 at Data File)

Ld_Case_TR : Case temperature of LW sensor translated into Black Body Emission (Unit: kW m-2) (No. 39)

Ld_Dome_TR : Similar to Ld_Case_TR but Dome temperature of the sensor (Unit: kW m-2) (No. 40)

K : Calibration Coefficient (Unit: mV W-1 m2)

C : Correction Coefficient for Secondary Emission from the dome of sensor.

(C = 2.0 - 3.0 is applicable.)

Upward Longwave Radiation can be computed similar to Downward LW.