EAST　TIBET

Interval:5sec

ID=D105：10、N-PAM：20、BJ：30

Output_Table  60.00 Min

| No. | Value |  | Processing | Unit |
| --- | --- | --- | --- | --- |
| 1 | ID | D105：10、N-PAM：20、BJ：30 |  |  |
| 2 | Year |  |  |  |
| 3 | Day | Julian day |  |  |
| 4 | Hour_minute |  |  |  |
| 5 | U_1 | Wind speed at top level | 10 minute average (50:05 – 60:00, 120data) | m/s |
| 6 | dd | Wind direction | 10 minute average (50:05 – 60:00, 120 data) | deg |
| 7 | U_1 | Wind speed at top level | 60 minute average (00:05 – 60:00, 720 data) | m/s |
| 8 | dd | Wind direction | 60 minute average (00:05 – 60:00, 720 data) | deg |
| 9 | U_1 | Maximum wind speed at top level | Maximum (00:05 – 60:00, 720 data) | m/s |
| 10 | U_2 | Wind speed at middle level | 10 minute average (50:05 – 60:00, 120 data) | m/s |
| 11 | U_2 | Wind speed at middle level | 60 minute average (00:05 – 60:00, 720 data) | m/s |
| 12 | U_3_10A | Wind speed at the lowest level | 10 minute average (00:05 – 10:00, 120 data) | m/s |
| 13 | U_3_20A | Wind speed at the lowest level | 10 minute average (10:05 – 20:00, 120 data) | m/s |
| 14 | U_3_30A | Wind speed at the lowest level | 10 minute average (20:05 – 30:00, 120 data) | m/s |
| 15 | U_3_40A | Wind speed at the lowest level | 10 minute average (30:05 – 40:00, 120 data) | m/s |
| 16 | U_3_50A | Wind speed at the lowest level | 10 minute average (40:05 – 50:00, 120 data) | m/s |
| 17 | U_3_60A | Wind speed at the lowest level | 10 minute average (50:05 – 60:00, 120 data) | m/s |
| 18 | T_1 | Air temperature at middle level | 10 minute average (50:05 – 60:00, 120 data) | degC |
| No. | Value |  | Processing | Unit |
| 19 | T_2 | Air temperature at the lowest level | 10 minute average (50:05 – 60:00, 120 data) | degC |
| 20 | Diff_T | Temperature difference T_1-T_2 | 10 minute average (50:05 – 60:00, 120 data) | degC |
| 21 | H_1 | Relative humidity at middle level | 10 minute average (50:05 – 60:00, 120 data) | % |
| 22 | H_1ref | Reference humidity at middle level | 10 minute average (50:05 – 60:00, 120 data) | % |
| 23 | H_2 | Relative humidity at the lowest level | 10 minute average (50:05 – 60:00, 120 data) | % |
| 24 | H_2ref | Reference humidity at the lowest level | 10 minute average (50:05 – 60:00, 120 data) | % |
| 25 | Ts_1 | Soil temperature (Sensor 1) | 10 minute average (50:05 – 60:00, 120 data) | DegC |
| 26 | Ts_2 | Soil temperature (Sensor 2) | 10 minute average (50:05 – 60:00, 120 data) | DegC |
| 27 | Ts_3 | Soil temperature (Sensor 3) | 10 minute average (50:05 – 60:00, 120 data) | DegC |
| 28 | Ts_4 | Soil temperature (Sensor 4) | 10 minute average (50:05 – 60:00, 120 data) | DegC |
| 29 | Ts_5 | Soil temperature (Sensor 5) | 10 minute average (50:05 – 60:00, 120 data) | DegC |
| 30 | Ts_6 | Soil temperature (Sensor 6) | 10 minute average (50:05 – 60:00, 120 data) | DegC |
| 31 | Sd | Downward shortwave radiation voltage of thermopile | 10 minute average (50:05 – 60:00, 120 data) | mVDC |
| 32 | Sd | Downward shortwave radiation voltage of thermopile | 60 minute average (00:05 – 60:00, 720 data) | mVDC |
| 33 | Su | Upward shortwave radiation voltage of thermopile | 10 minute average (50:05 – 60:00, 120 data) | mVDC |
| 34 | Su | Upward shortwave radiation voltage of thermopile | 60 minute average (00:05 – 60:00, 720 data) | mVDC |
| 35 | Ld_Rnet | Downward longwave radiation Voltage of thermopile | 10 minute average (50:05 – 60:00, 120 data) | mVDC |
| 36 | Ld_CaseTR | Inside temperature of downward longwave sensor translated into black body emission | 10 minute average (50:05 – 60:00, 120 data) | kWm-2 |
| No. | Value |  | Average(or Instantaneous) | Unit |
| 37 | Ld_DomeTR | Same as ‘Ld_CaseTR’ (No.36) but dome temperature | 10 minute average (50:05 – 60:00, 120 data) | kWm-2 |
| 38 | Ld_Rnet | Downward longwave radiation Voltage of thermopile | 60 minute average (00:05 – 60:00, 720 data) | mVDC |
| 39 | Ld_CaseTR | Inside temperature of downward longwave sensor translated into black body emission | 60 minute average (00:05 – 60:00, 720 data) | kWm-2 |
| 40 | Ld_DomeTR | Same as ‘Ld_CaseTR’ (No.39) but dome temperature | 60 minute average (00:05 – 60:00, 720 data) | kWm-2 |
| 41 | Lu_Rnet | Upward longwave radiation Voltage of thermopile | 10 minute average (50:05 – 60:00, 120 data) | mVDC |
| 42 | Lu_CaseTR | Inside temperature of upward longwave sensor translated into black body emission | 10 minute average (50:05 – 60:00, 120 data) | kWm-2 |
| 43 | Lu_DomeTR | Same as ‘Lu_CaseTR’ (No.42) but dome temperature | 10 minute average (50:05 – 60:00, 120 data) | kWm-2 |
| 44 | Lu_Rnet | Upward longwave radiation Voltage of thermopile | 60 minute average (00:05 – 60:00, 720 data) | mVDC |
| 45 | Lu_CaseTR | Inside temperature of upward longwave sensor translated into black body emission | 60 minute average (00:05 – 60:00, 720 data) | kWm-2 |
| 46 | Lu_DomeTR | Same as ‘Lu_CaseTR’ (No.45) but dome temperature | 60 minute average (00:05 – 60:00, 720 data) | kWm-2 |
| 47 | G_1 | Soil heat flux of sensor 1 in voltage | 10 minute average (50:05 – 60:00, 120 data) | mVDC |
| 48 | G_1 | Soil heat flux of sensor 1 in voltage | 60 minute average (00:05 – 60:00, 720 data) | mVDC |
| 49 | G_2 | Soil heat flux of sensor 2 in voltage | 10 minute average (50:05 – 60:00, 120 data) | mVDC |
| 50 | G_2 | Soil heat flux of sensor 2 in voltage | 60 minute average (00:05 – 60:00, 720 data) | mVDC |
| 51 | Trad | Radiometric surface temperature In degC | 10 minute average (50:05 – 60:00, 120 data) | degC |
| 52 | Trad_Tr | Radiometric surface temperature Translated into blacj body emission | 10 minute avetage (50:05 – 60:00, 120 data) | kWm-2 |
| 53 | Trad_Tr | Radiometric surface temperature Translated into blacj body emission | 60 minute avetage (00:05 – 60:00, 720 data) | kWm-2 |
| 54 | SW_1 | Soil moisture content of sensor 1 | Instantaneous at 48:00 | % |
| No. | Value |  | Processing | Unit |
| 55 | SW_2 | Soil moisture content of sensor 2 | Instantaneous at 48:00 | % |
| 56 | P | Atmospheric pressure | Instantaneous at 59:50 | hPa |
| 57 | Rain_10T | Accumulated precipitation | 10 minute accumulated value (00:05 – 10:00) | pulse |
| 58 | Rain_20T | Accumultated precipitation | 10 minute accumulated value (10:05 – 20:00) | pulse |
| 59 | Rain_30T | Accumultated precipitation | 10 minute accumulated value (10:05 – 20:00) | pulse |
| 60 | Rain_40T | Accumultated precipitation | 10 minute accumulated value (10:05 – 20:00) | pulse |
| 61 | Rain_50T | Accumultated precipitation | 10 minute accumulated value (10:05 – 20:00) | pulse |
| 62 | Rain_60T | Accumultated precipitation | 10 minute accumulated value (10:05 – 20:00) | pulse |
| 63 | Snow | Depth of snow | Instantaneous value at 60:00 | M |